Kepler Data Release 9 Notes

KSCI-19049-001

Data Analysis Working Group (DAWG)

Jessie Christiansen (Editor)

Pavel Machalek (Editor)

Data Release 9 for Quarter Q6

| Q.m |  | First Cadence MJD midTime | Last Cadence MJD midTime | First Cadence UT midTime | Last Cadence UT midTime | Num CINs | Start CIN | End CIN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | LC |  |  |  |  |  |  |  |
| 0 | SC | 54953.028 | 54962.754 | 5/2/09 0:40 | 5/11/09 18:05 | 14280 |  |  |
| 0 | LC | 54953.038 | 54962.744 | 5/2/09 0:54 | 5/11/09 17:51 | 476 |  |  |
| 1 | SC | 54964.001 | 54997.491 | 5/13/09 0:01 | 6/15/09 11:47 | 49170 |  |  |
| 1 | LC | 54964.011 | 54997.481 | 5/13/09 0:15 | 6/15/09 11:32 | 1639 |  |  |
| 2.1 | SC | 55002.008 | 55032.800 | 6/20/09 0:10 | 7/20/09 19:12 | 45210 |  |  |
| 2.2 | SC | 55032.822 | 55062.797 | 7/20/09 19:42 | 8/19/09 19:07 | 44010 |  |  |
| 2.3 | SC | 55063.860 | 55090.975 | 8/20/09 20:38 | 9/16/09 23:23 | 39810 |  |  |
| 2 | LC | 55002.018 | 55090.965 | 6/20/09 0:25 | 9/16/09 23:09 | 4354 |  |  |
| 3 | LC | 55092.7222 | 55181.9966 | 9/18/09 17:19 | 12/16/09 23:55 | 4370 |  |  |
| 3.1 | SC | 55092.7123 | 55123.0555 | 9/18/09 17:05 | 10/19/09 1:19 | 44550 |  |  |
| 3.2 | SC | 55123.9144 | 55153.9511 | 10/19/09 21:56 | 11/18/09 22:49 | 44100 |  |  |
| 3.3 | SC | 55156.0156 | 55182.0065 | 11/21/09 0:22 | 12/17/09 0:09 | 38160 |  |  |
| 0 | LC | 54953.038 | 54962.744 | 5/2/09 0:54 | 5/11/09 17:51 | 476 |  |  |
| 0 | SC | 54953.028 | 54962.754 | 5/2/09 0:40 | 5/11/09 18:05 | 14280 |  |  |
| 1 | LC | 54964.011 | 54997.481 | 5/13/09 0:15 | 6/15/09 11:32 | 1639 |  |  |
| 1 | SC | 54964.001 | 54997.491 | 5/13/09 0:01 | 6/15/09 11:47 | 49170 |  |  |
| 6 | LC | 55371.9473 | 55461.7939 | 06/24/10 22:44 | 09/22/10 19:03 | 4397 | 21069 | 25466 |
| 6.1 | SC | 55371.9375 | 55399.0317 | 06/24/10 22:29 | 07/22/10 00:45 | 39779 | 620530 | 660309 |
| 6.2 | SC | 55399.8702 | 55430.7855 | 07/22/10 20:53 | 08/22/10 18:51 | 45389 | 661540 | 706929 |
| 6.3 | SC | 55431.6853 | 55461.8037 | 08/23/10 16:26 | 09/22/10 19:17 | 44219 | 708250 | 752469 |

Prepared by: _______________________________________ Date __________

Jessie Christiansen, Kepler Science Office, for the DAWG (next page)

Prepared by: _______________________________________ Date __________

Pavel Machalek, Kepler Science Office, for the DAWG (next page)

Approved by: _______________________________________ Date __________

Jeffrey Van Cleve, Editor-in-Chief

Approved by: _______________________________________ Date __________

Jon Jenkins, Co-I for Data Analysis & DAWG Lead

Approved by: _______________________________________ Date __________

Michael R. Haas, Science Office Director

Document Control

Ownership

This document is part of the Kepler Project Documentation that is controlled by the Kepler Project Office, NASA/Ames Research Center, Moffett Field, California.

Control Level

This document will be controlled under KPO @ Ames Configuration Management system. Changes to this document shall be controlled.

Physical Location

The physical location of this document will be in the KPO @ Ames Data Center.

Distribution Requests

To be placed on the distribution list for additional revisions of this document, please address your request to the Kepler Science Office:

Michael R. Haas

Kepler Science Office Director

MS 244-30

NASA Ames Research Center

Moffett Field, CA 94035-1000

Michael.R.Haas@nasa.gov

# The Data Characteristics Handbook is the collective effort of the Data Analysis Working Group (DAWG), composed of Science Office (SO), Science Operations Center (SOC), Guest Observer Office (GO), and Science Team (ST) members as listed below:

# Jon Jenkins*, Chair

Doug Caldwell*, Co-Chair

Allen, Christopher L.

Bryson, Stephen T.

Christiansen, Jessie L.

Clarke, Bruce D.

Cote, Miles T.

Fanelli, Michael N.

Gilliland*, Ron (STSci)

Girouard, Forrest

Haas, Michael R.

Hall, Jennifer

Ibrahim, Khadeejah

Kinemuchi, Karen

Klaus, Todd

Kolodziejczak, Jeff (MSFC)

Li, Jie

Machalek, Pavel

McCauliff, Sean D.

Middour, Christopher K.

Morris, Rob

Mullally, Fergal

Quintana, Elisa V.

Rowe, Jason

Seader, Shawn

Smith, Jeffrey Claiborne

Still, Martin

Tenenbaum, Peter G.

Thompson, Susan E.

Twicken, Joe

Uddin, Akm Kamal

Van Cleve, Jeffrey

Wohler, Bill

# *Science Team

# Table of Contents

Prefatory Admonition to Users	6

1.	Introduction	7

1.1	Dates, Cadence numbers, and units	7

2.	Release Description	8

3.	Evaluation of Performance	9

3.1	Overall	9

4.	Historical Events	11

4.1	Kepler mission timeline to date	11

4.2	Safe Mode	11

4.3	Loss of Fine Point	11

4.4	Attitude Tweaks	12

4.5	Variable FGS Guide Stars	12

4.6	Module 3 Failure	12

5.	Ongoing Phenomena	13

5.1	Image Motion	13

5.2	Focus Drift and Jitter	14

5.3	Momentum Desaturation	14

5.4	Reaction Wheel Zero Crossings	16

5.5	Downlink Earth Point	16

5.6	Manually Excluded Cadences	16

5.7	Incomplete Apertures Give Flux and Feature Discontinuities at Quarter Boundaries	16

5.8	Argabrightening	17

5.9	Background Time Series	19

5.10	Pixel Sensitivity Dropouts	19

5.11	Short Cadence Requantization Gaps	19

5.12	Spurious Frequencies in SC Data	19

5.13	Anomaly Summary Table	20

6.	Time and Time Stamps	22

7.	Contents of Supplement	23

8.	References	25

# Prefatory Admonition to Users

The corrected light-curve product generated by the PDC (Pre-search Data Conditioning) pipeline module is designed to enable the Kepler planetary transit search. Although significant effort has been expended to preserve the natural variability of targets in the corrected light curves in order to enable astrophysical exploitation of the Kepler data, it is not possible to perfectly preserve general stellar variability on long timescales with amplitudes comparable to or smaller than the instrumental systematics, and PDC currently is known to remove or distort astrophysical features in a subset of the corrected light curves. In those cases where PDC fails, or where the requirements of an astrophysical investigation are in conflict with those for transit planet search, the investigator should use the ‘raw’ light-curve product, for which basic calibration has been performed but correction for instrumental systematics has not, instead of the PDC (‘corrected’) light-curve product. Where appropriate, the investigator can then use the ancillary engineering data and image motion time series provided in the relevant Data Release Notes Supplement/s for systematic error correction. Investigators are strongly encouraged to study the Data Characteristics Handbook and Data Release Notes for any data sets they intend to use. The Science Office advises against publication of results based on Kepler light curves without careful consideration and due diligence by the end user, and dialog with the Science Office or Guest Observer Office where appropriate.

Users are encouraged to notice and document artifacts, either in the raw or processed data, and report them to the Science Office at kepler-scienceoffice@lists.nasa.gov.

Users who neglect this Admonition risk seeing their works crumble into ruin before their time.

# Introduction

These Data Release Notes provide information specific to the quarter of data currently being released.  They have been drastically shortened relative to previous versions.  The companion text has been moved to the Kepler Data Characterization Handbook (KSCI-19040).  The section numbers and titles are identical in these Notes and that Handbook to assist the reader.

## Dates, Cadence numbers, and units

Table 1:  Contents of Release 9. CIN is the cadence interval number. All Release 9 cadence data were processed under KSOP-652 with SOC Pipeline 6.2, revision number r40414.

| Q.m |  | First Cadence MJD midTime | Last Cadence MJD midTime | First Cadence UT midTime | Last Cadence UT midTime | Num CINs | Start CIN | End CIN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | LC |  |  |  |  |  |  |  |
| 0 | SC | 54953.028 | 54962.754 | 5/2/09 0:40 | 5/11/09 18:05 | 14280 |  |  |
| 0 | LC | 54953.038 | 54962.744 | 5/2/09 0:54 | 5/11/09 17:51 | 476 |  |  |
| 1 | SC | 54964.001 | 54997.491 | 5/13/09 0:01 | 6/15/09 11:47 | 49170 |  |  |
| 1 | LC | 54964.011 | 54997.481 | 5/13/09 0:15 | 6/15/09 11:32 | 1639 |  |  |
| 2.1 | SC | 55002.008 | 55032.800 | 6/20/09 0:10 | 7/20/09 19:12 | 45210 |  |  |
| 2.2 | SC | 55032.822 | 55062.797 | 7/20/09 19:42 | 8/19/09 19:07 | 44010 |  |  |
| 2.3 | SC | 55063.860 | 55090.975 | 8/20/09 20:38 | 9/16/09 23:23 | 39810 |  |  |
| 2 | LC | 55002.018 | 55090.965 | 6/20/09 0:25 | 9/16/09 23:09 | 4354 |  |  |
| 3 | LC | 55092.7222 | 55181.9966 | 9/18/09 17:19 | 12/16/09 23:55 | 4370 |  |  |
| 3.1 | SC | 55092.7123 | 55123.0555 | 9/18/09 17:05 | 10/19/09 1:19 | 44550 |  |  |
| 3.2 | SC | 55123.9144 | 55153.9511 | 10/19/09 21:56 | 11/18/09 22:49 | 44100 |  |  |
| 3.3 | SC | 55156.0156 | 55182.0065 | 11/21/09 0:22 | 12/17/09 0:09 | 38160 |  |  |
| 0 | LC | 54953.038 | 54962.744 | 5/2/09 0:54 | 5/11/09 17:51 | 476 |  |  |
| 0 | SC | 54953.028 | 54962.754 | 5/2/09 0:40 | 5/11/09 18:05 | 14280 |  |  |
| 1 | LC | 54964.011 | 54997.481 | 5/13/09 0:15 | 6/15/09 11:32 | 1639 |  |  |
| 1 | SC | 54964.001 | 54997.491 | 5/13/09 0:01 | 6/15/09 11:47 | 49170 |  |  |
| 6 | LC | 55371.9473 | 55461.7939 | 06/24/10 22:44 | 09/22/10 19:03 | 4397 | 21069 | 25466 |
| 6.1 | SC | 55371.9375 | 55399.0317 | 06/24/10 22:29 | 07/22/10 00:45 | 39779 | 620530 | 660309 |
| 6.2 | SC | 55399.8702 | 55430.7855 | 07/22/10 20:53 | 08/22/10 18:51 | 45389 | 661540 | 706929 |
| 6.3 | SC | 55431.6853 | 55461.8037 | 08/23/10 16:26 | 09/22/10 19:17 | 44219 | 708250 | 752469 |

# Release Description

No changes from the Data Characteristics Handbook.

# Evaluation of Performance

## Overall

Figure 1:  6.5 hr Temporal Median (TM) of the Quarter 6 CDPP time series calculated by the TPS pipeline module for stars between 9th and 13.5th magnitude. The 6 hr TMCDPPs have been divided by sqrt(13/12) = 1.041 to approximate 6.5 hr TMCDPPs. Stars on the planetary target list with Kepler Magnitude < 13.5 and log g > 4, which are almost certainly dwarf stars, are shown as green  +'s; other stars are marked with blue +'s.

Table 2:  Aggregate Statistics for the TMCDPPs plotted in  Figure 1:  6.5 hr Temporal Median (TM) of the Quarter 6 CDPP time series calculated by the TPS pipeline module for stars between 9th and 13.5th magnitude. The 6 hr TMCDPPs have been divided by sqrt(13/12) = 1.041 to approximate 6.5 hr TMCDPPs. Stars on the planetary target list with Kepler Magnitude < 13.5 and log g > 4, which are almost certainly dwarf stars, are shown as green  +'s; other stars are marked with blue +'s.. Column Definitions:  (1) Kepler Magnitude at center of bin.  Bins are +/- 0.25 mag, for a bin of width 0.5 mag centered on this value. (2) Number of dwarfs (log g > 4) in bin. (3) 10th percentile TMCDPP for dwarfs in bin. (4) Median TMCDPP for dwarfs in bin. (5) Number of all stars in bin. (6) 10th percentile TMCDPP of all observed stars in bin. (7) Median TMCDPP for all stars in bin. (8) Simplified noise model CDPP.

| Kepler mag at bin center | Number of dwarfs in bin | 10th percentile CDPP, dwarfs | Median CDPP, dwarfs | Number of all stars in bin | 10th percentile CDPP, all stars | Median CDPP, all stars | Lower envelope of model CDPP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | 31 | 9.8 | 37 | 180 | 13.2 | 89.5 | 3.8 |
| 10 | 160 | 12.1 | 34.6 | 567 | 14.7 | 100.3 | 6 |
| 11 | 607 | 18.5 | 33.8 | 1707 | 21.4 | 94.2 | 9.5 |
| 12 | 2169 | 24.9 | 39.2 | 4385 | 27 | 69.5 | 15.2 |
| 13 | 6767 | 36.4 | 50.9 | 10645 | 38.2 | 64.7 | 24.4 |

# Historical Events

In this Section, we discuss cadences that may be lost to high-precision photometry due to planned or unplanned spacecraft events.

## Kepler mission timeline to date

Figure 2: Kepler Mission Timeline as of end of Q6.

## Safe Mode

No changes from the Data Characteristics Handbook.

## Loss of Fine Point

No changes from the Data Characteristics Handbook.

## Attitude Tweaks

Figure 3: Attitude Error in Quarter 6, calculated by PAD using Long Cadence data.

## Variable FGS Guide Stars

No changes from the Data Characteristics Handbook.

## Module 3 Failure

No changes from the Data Characteristics Handbook.

# Ongoing Phenomena

## Image Motion

Figure 4:  Mod out 2.1 Center motion time series. The gaps are monthly Earth contacts.

## Focus Drift and Jitter

Figure 5:  Correlation of variation in PRF width with various spacecraft temperatures, demonstrating the seasonal nature of focus and PRF changes.

## Momentum Desaturation

Table 3:  Momentum dumps in Q6 and the corresponding Long and Short Cadences. CIN = cadence interval number, RCI = relative cadence index.

LC

CIN     RCI   Date(MJD)

21150     82 55373.60245

21296    228 55376.58576

21442    374 55379.56906

21588    520 55382.55237

21734    666 55385.53567

21880    812 55388.51898

22026    958 55391.50229

22172   1104 55394.48559

22318   1250 55397.46890

22393   1325 55399.00142

22538   1470 55401.96429

22684   1616 55404.94760

22830   1762 55407.93090

22976   1908 55410.91421

23122   2054 55413.89752

23268   2200 55416.88082

23414   2346 55419.86413

23560   2492 55422.84743

23706   2638 55425.83074

23852   2784 55428.81405

23947   2879 55430.75524

24093   3025 55433.73852

24239   3171 55436.72183

24385   3317 55439.70514

24531   3463 55442.68844

24677   3609 55445.67175

24822   3754 55448.63462

24968   3900 55451.61793

25114   4046 55454.60123

25260   4192 55457.58454

25406   4338 55460.56784

25465   4397 55461.77343

SC

CIN    RCI    Date(MJD)

(Month 1)

622970   2441 55373.59938

622971   2442 55373.60007

627350   6821 55376.58269

627351   6822 55376.58337

631730  11201 55379.56600

631731  11202 55379.56668

636110  15581 55382.54930

636111  15582 55382.54998

640490  19961 55385.53261

640491  19962 55385.53329

644870  24341 55388.51592

644871  24342 55388.51660

649250  28721 55391.49922

653630  33101 55394.48253

658010  37481 55397.46583

660260  39731 55398.99835

(Month 2)

664610   3071 55401.96123

668990   7451 55404.94453

673370  11831 55407.92784

677750  16211 55410.91115

682130  20591 55413.89445

686510  24971 55416.87776

690890  29351 55419.86106

695270  33731 55422.84437

699650  38111 55425.82768

704030  42491 55428.81098

706880  45341 55430.75217

(Month 3)

711260   3011 55433.73546

715640   7391 55436.71876

720020  11771 55439.70207

720021  11772 55439.70275

724400  16151 55442.68538

728780  20531 55445.66868

733130  24881 55448.63156

733131  24882 55448.63224

737510  29261 55451.61486

737511  29262 55451.61554

741890  33641 55454.59817

741891  33642 55454.59885

746270  38021 55457.58147

746271  38022 55457.58215

750650  42401 55460.56478

750651  42402 55460.56546

752420  44171 55461.77036

## Reaction Wheel Zero Crossings

Table 4: Zero crossing events in Q6, defined as the time from first to last zero crossing in the event, rounded to the nearest Cadence.  The corresponding cadence numbers for SC are in the supplement.

| Event# | MJD Start | MJD End | CIN Start | CIN End | RCI Start | RCI End | NumTLM Samp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 55379.569 | 55379.957 | 21442 | 21461 | 374 | 393 | 280 |
| 2 | 55382.552 | 55382.695 | 21588 | 21595 | 520 | 527 | 103 |
| 3 | 55388.499 | 55388.601 | 21879 | 21884 | 811 | 816 | 55 |
| 4 | 55397.469 | 55397.510 | 22318 | 22320 | 1250 | 1252 | 29 |
| 5 | 55410.894 | 55410.935 | 22975 | 22977 | 1907 | 1909 | 16 |

## Downlink Earth Point

No changes from the Data Characteristics Handbook.

## Manually Excluded Cadences

No changes from the Data Characteristics Handbook.

## Incomplete Apertures Give Flux and Feature Discontinuities at Quarter Boundaries

No changes from the Data Characteristics Handbook.

## Argabrightening

Table 5:  Q6 LC Argabrightening Events with amplitude TMAD > 10, and occurring on a number of channels TMCE > 10. The columns are (1) CIN = Cadence Interval Number for Argabrightening cadences, (2) RCI = relative cadence index for Argabrightening cadences, (3) Date = Arg cadence mid-Times, MJD, (4) Mean Argabrightening statistic over Channels of Arg Event <SArg>FPA (5) N_chan = Channels exceeding threshold in Arg cadence, (6) N_pipe = Channels exceeding default (pipeline) threshold in Arg cadence. MAD is calculated on a channel-by-channel basis.

CIN     RCI   Mid-Times(MJD) <SArg> N_chan  N_pipe

21601    533 55382.81801   17.4  68   0

21724    656 55385.33134    7.2  18   0

21836    768 55387.61990    7.6  19   0

22014    946 55391.25708    8.5  26   0

22169   1101 55394.42429    8.2  23   0

22296   1228 55397.01936    7.8  21   0

23130   2062 55414.06099    8.3  27   0

23465   2397 55420.90624   62.4  69  13

23466   2398 55420.92668   15.4  27   1

23616   2548 55423.99172  100.0  80  36

23944   2876 55430.69394    4.9  11   0

23945   2877 55430.71437    7.4  18   0

23946   2878 55430.73481   10.5  30   0

23948   2880 55430.77567   18.9  59   0

24061   2993 55433.08465   14.0  56   0

24386   3318 55439.72557    7.8  16   0

24427   3359 55440.56335   36.9  79   0

24470   3402 55441.44199   66.3  80  13

24640   3572 55444.91570    6.8  22   0

24796   3728 55448.10335    3.3  13   0

24994   3926 55452.14920    7.4  12   0

25111   4043 55454.53993   10.0  34   0

25334   4266 55459.09662   18.2  67   0

Table 6: Q6 SC Argabrightening Events with amplitude TMAD > 10, and occurring on a number of channels TMCE > 10. The columns have the same meanings as Table 5:  Q6 LC Argabrightening Events with amplitude TMAD > 10, and occurring on a number of channels TMCE > 10. The columns are (1) CIN = Cadence Interval Number for Argabrightening cadences, (2) RCI = relative cadence index for Argabrightening cadences, (3) Date = Arg cadence mid-Times, MJD, (4) Mean Argabrightening statistic over Channels of Arg Event <SArg>FPA (5) N_chan = Channels exceeding threshold in Arg cadence, (6) N_pipe = Channels exceeding default (pipeline) threshold in Arg cadence. MAD is calculated on a channel-by-channel basis. Note consecutive detections of the largest events. A horizontal line separates the 3 Months of the Quarter. The relative cadence index (RCI) is reset at the start of each Month.

CIN     RCI   Mid-Times(MJD) <SArg>  N_chan  N_pipe

636501  15972 55382.81562   17.6  69   0

640187  19658 55385.32623    8.4  23   0

648880  28351 55391.24721    9.3  30   0

665777   4238 55402.75609    7.1  15   0

692439  30900 55420.91612   71.1  66  42

692440  30901 55420.91680   15.7  22   6

696948  35409 55423.98729   44.0  76  16

696949  35410 55423.98797   66.0  69  37

710291   2042 55433.07545    8.3  21   0

720044  11795 55439.71842    6.0  12   2

721281  13032 55440.56096   33.8  77   3

722571  14322 55441.43961   12.7  43   0

722572  14323 55441.44029   17.2  56   1

722573  14324 55441.44097   29.3  73   4

727678  19429 55444.91809    6.3  20   0

748502  40253 55459.10173    9.7  32   0

## Background Time Series

Figure 6:  Background time series for Q6 showing the average over all the modules, and the modules furthest from (2.4) and nearest to (24.4) the Galactic plane. The narrow spikes are Argabrightening events.

## Pixel Sensitivity Dropouts

No changes from the Data Characteristics Handbook.

## Short Cadence Requantization Gaps

No changes from the Data Characteristics Handbook.

## Spurious Frequencies in SC Data

No changes from the Data Characteristics Handbook.

## Anomaly Summary Table

Machine readable CINs for the anomalies below are in the Data Supplement.

Table 7: Anomaly Summary Table for Long and Short Cadences

| LC CIN |  |  |  |
| --- | --- | --- | --- |
| Start | End | Anomaly Type | Note |
| 21069 | 21069 | EARTH_POINT | Monthly science data downlink |
| 22395 | 22436 | EARTH_POINT | Monthly science data downlink |
| 23949 | 23992 | EARTH_POINT | Monthly science data downlink |
| 23465 | 23466 | ARGABRIGHTENING | See Section 5.8. |
| 23616 | 23616 | ARGABRIGHTENING |  |
| 24470 | 24470 | ARGABRIGHTENING |  |

| SC CIN |  |  |  |
| --- | --- | --- | --- |
| Start | End | Anomaly Type | Note |
| 622969 | 622980 | COARSE POINT | See Section 5.3 for Momentum desaturations. |
| 627349 | 627360 | COARSE POINT |  |
| 631729 | 631740 | COARSE POINT |  |
| 636109 | 636120 | COARSE POINT |  |
| 640489 | 640500 | COARSE POINT |  |
| 644869 | 644880 | COARSE POINT |  |
| 649249 | 649260 | COARSE POINT |  |
| 653629 | 653640 | COARSE POINT |  |
| 658009 | 658020 | COARSE POINT |  |
| 660259 | 660270 | COARSE POINT |  |
| 664609 | 664620 | COARSE POINT |  |
| 668989 | 669000 | COARSE POINT |  |
| 673369 | 673380 | COARSE POINT |  |
| 677749 | 677760 | COARSE POINT |  |
| 682129 | 682140 | COARSE POINT |  |
| 686509 | 686520 | COARSE POINT |  |
| 690889 | 690900 | COARSE POINT |  |
| 695269 | 695280 | COARSE POINT |  |
| 696949 | 696949 | COARSE POINT |  |
| 699649 | 699660 | COARSE POINT |  |
| 704029 | 704040 | COARSE POINT |  |
| 706879 | 706890 | COARSE POINT |  |
| 692439 | 692440 | ARGABRIGHTENING | See Section 5.8 |
| 696948 | 696949 | ARGABRIGHTENING |  |
| 711259 | 711270 | COARSE POINT | See Section 5.3 for Momentum desaturations. |
| 715639 | 715650 | COARSE POINT |  |
| 720019 | 720030 | COARSE POINT |  |
| 724399 | 724410 | COARSE POINT |  |
| 728779 | 728790 | COARSE POINT |  |
| 733129 | 733140 | COARSE POINT |  |
| 737509 | 737520 | COARSE POINT |  |
| 741889 | 741900 | COARSE POINT |  |
| 746269 | 746280 | COARSE POINT |  |
| 750649 | 750660 | COARSE POINT |  |
| 750970 | 751029 | COARSE POINT |  |
| 752419 | 752430 | COARSE POINT |  |

# Time and Time Stamps

No changes from the Data Characteristics Handbook.

# Contents of Supplement

The Supplement is available as a full package (DataReleaseNotes_09_SupplementFull.tar), which contains the files described below.

Pipeline Instance Detail Reports

Q6_LC_r6.2_ksop652_PA_+PDC_with_mpe_asrun_Pipeline_Instance_Detail_Report_110104.txt

Q6_SCM1_r6.2_ksop652_with_mpe_as-run_Pipeline_Instance_Detail_Report_110104.txt

Q6_SCM2_r6.2_ksop652_with_mpe_as-run_Pipeline_Instance_Detail_Report_110104.txt

Q6_SCM3_r6.2_ksop652_with_mpe_as-run_Pipeline_Instance_Detail_Report_110104.txt

Data Anomaly Types

DataAnomalyTypes_Q6_LC_PID3197_Summary.txt

DataAnomalyTypes_Q6M1_SC_PID3217_Summary.txt

DataAnomalyTypes_Q6M2_SC_PID3237_Summary.txt

DataAnomalyTypes_Q6M3_SC_PID3277_Summary.txt

Mod.out Central Motion

Q6_central_row_motion.txt

Q6_central_column_motion.txt

Average LDE board Temperature

Q6_LDE_averageBoardTemp.txt

Q6_TH12LVAT_MJD_gap.txt

Q6_TH1RW34T_MJD_gap.txt

Background Time Series

Q6_SCM1_background.txt

Q6_SCM2_background.txt

Q6_SCM3_background.txt

Q6_LC_background.txt

Flight System Events

Argabrightening Detections

ArgAgg_Q6_LC_PID3197_MADT010_MCT10_Summary.txt

ArgAgg_Q6M1_SC_PID3557_MADT010_MCT10_Summary.txt

ArgAgg_Q6M2_SC_PID3577_MADT010_MCT10_Summary.txt

ArgAgg_Q6M3_SC_PID3597_MADT010_MCT10_Summary.txt

Out of Fine Point Cadence Lists

Q6M1_SC_isNotFinePoint.txt

Q6M2_SC_isNotFinePoint.txt

Q6M3_SC_isNotFinePoint.txt

Q6_LC_isNotFinePoint.txt

Zero Crossing Events

Q6_Dec21_SC_ZeroCrossings.txt

Q6_Dec21_LC_ZeroCrossings.txt

Short Supplement Package

The Supplement also contains a short package suitable for emailing (DataReleaseNotes_09_SupplementSmall.tar). The small package does not contain the following files:

13Dec_SC_Q6M1_background.txt

13Dec_SC_Q6M2_background.txt

13Dec_SC_Q6M3_background.txt

Dec13_LC_Q6_background.txt

13Dec_Q6_LC_central_row_motion.txt

13Dec_Q6_LC_central_column_motion.txt

Q6_TH12LVAT_MJD_gap.txt

Q6_TH1RW34T_MJD_gap.txt

# References

No changes from Data Characteristics Handbook.